# Gas Leak Detection System

Prof. Archan Chaudhari

Department of Instrumentation

and control engineering

Vishwakarma Institute of Technology Pune, Maharashtra, India

archana.chaudhari1@vit.edu

Nishant A. Pawar

Department of Instrumentation

and control engineering

Vishwakarma Institute of Technology Pune, Maharashtra, India nishant.pawar23@vit.edu

Devanshu K. Kamdi

Department of Instrumentation

and control engineering

Vishwakarma Institute of Technology Pune, Maharashtra, India

devanshu.kamdi23@vit.edu

Aradhay V. Kopulwar

Department of Instrumentation

and control engineering

Vishwakarma Institute of Technology Pune, Maharashtra, India

aradhay.kopulwar23@vit.edu 
Romila C..Katkar

Department of Instrumentation

and control engineering

Vishwakarma Institute of Technology Pune, Maharashtra, India

romila.katkar23@vit.edu

Abstract—This research presents the design and implementation of an IoT-based Gas Leak Detection and Prevention System using the ESP32 microcontroller. The system leverages the MQ5 gas sensor for LPG detection and an infrared flame sensor to identify open flames. A relay-controlled solenoid valve actively shuts off the gas supply in case of leakage, while a buzzer and LED provide immediate local alerts. Integration with a Telegram bot enables real-time notifications and remote control via mobile devices. Unlike traditional systems, this setup provides both automation and remote accessibility, ensuring enhanced safety for domestic and commercial kitchens. The system was tested under simulated leakage conditions and successfully triggered protective actions, making it a reliable and cost-effective solution for early hazard prevention.

Keywords - ESP32, Gas Leak Detection, MQ5 Sensor, Solenoid Valve, Relay Module, IoT, Telegram Bot, LPG Safety, Flame Sensor,

# Introduction

The widespread use of Liquefied Petroleum Gas (LPG) for domestic and small-scale commercial cooking has made kitchen safety a critical concern. Two of the most prevalent hazards in such environments are accidental gas leaks and open flames left unattended—often resulting from forgotten or malfunctioning gas stoves. Both scenarios carry the potential for catastrophic outcomes such as fire, explosion, and loss of life or property.

To address these challenges, this research presents the design and implementation of a smart, dual-sensor-based Gas Leak and Flame Detection and Prevention System. At its core, the system utilizes an ESP32 microcontroller integrated with two key sensors: the MQ5 sensor for LPG concentration monitoring and an IR flame sensor for detecting exposed flames. These components work in tandem to ensure comprehensive safety coverage in real-time.

When a gas leak is detected—i.e., the MQ5 sensor registers a concentration above a specified threshold—the system automatically activates a relay to shut the solenoid valve, thereby cutting off gas supply. Concurrently, visual (LED) and auditory (buzzer) alerts are triggered to warn nearby occupants. This process is autonomous and immediate, minimizing human reaction time.

In the case of flame detection, however, the system adopts a controlled response. Recognizing that a flame may be present intentionally (e.g., cooking), it does not trigger the solenoid valve automatically. Instead, a manual control interface is provided via a Telegram bot, allowing the user to decide whether to cut off the gas supply. This ensures that false positives do not disrupt normal kitchen operations, while still maintaining safety in cases of unattended or suspicious flames.

The entire system is IoT-enabled through Telegram integration, allowing real-time alerts, 15-minute periodic safety updates during hazardous conditions, and remote control of the solenoid valve—all from a user’s mobile device. By combining intelligent sensing, automation, and remote access, this system provides an affordable and effective kitchen safety solution that blends hardware reliability with smart control.`

# LITERATURE REVIEW

Ensuring safety from gas leaks and unintended kitchen fires has been a growing concern, especially in urban households. Conventional gas leakage detectors relied on standalone alarms, but their lack of remote accessibility and control limited their effectiveness. Recent advances in embedded systems and IoT have enabled the development of intelligent safety mechanisms that allow real-time detection, notification, and remote control [1][2].

The MQ-5 gas sensor has been widely adopted in gas detection systems due to its sensitivity to LPG, methane, and hydrogen. Its integration with microcontrollers like Arduino and ESP32 has been documented in various studies [3][4]. Meanwhile, IR flame sensors have become a popular choice for low-cost flame detection applications, offering reliable identification of fire by sensing infrared radiation [5][6].

Modern systems often use the ESP32 due to its built-in Wi-Fi and high processing power. Researchers have developed LPG leak detectors using ESP32 combined with mobile applications like Blynk for real-time data monitoring and control [7][8]. Others have explored integrating fire detection with gas sensors, building hybrid systems that improve emergency response times [9][10].

Cloud platforms and messaging APIs such as Telegram have also been incorporated to provide instant notifications and control mechanisms via smartphones, enhancing user interaction and safety [11][12][13]. Additionally, literature highlights the role of mobile-based dashboards for real-time monitoring [14], and how smart relays and solenoid valves are used to automatically shut off gas supply when threats are detected [15][16].

Review studies show that systems using multiple sensors and IoT platforms perform better in detecting abnormal events early and improving user trust [17]. Some have even begun to include machine learning and image processing techniques for visual flame detection, although they often require more computational resources [18][19].

Overall, the integration of gas and flame detection, remote monitoring, and actuator control forms a comprehensive safety net that is both scalable and cost-effective for real-world deployment [20].

# Tools and technology

- 1.  Hardware Components

ESP32: The ESP32 microcontroller was selected for its superior processing power, built-in Wi-Fi and Bluetooth capabilities, and versatility in IoT projects. It is the central component of the system, processing data from sensors and controlling connected devices such as the solenoid valve.

MQ-5 Gas Sensor: This sensor is responsible for detecting LPG, methane, and other combustible gases. The analog signal from the sensor indicates the concentration of gas in the environment, allowing the system to detect any gas leakages.

IR Flame Sensor: The IR flame sensor detects infrared radiation from flames. This sensor ensures the system can respond to situations where a gas stove might be left on, contributing to fire hazards.

Relay Module (with Optocoupler): The relay module acts as a switch to control the solenoid valve. It uses an optocoupler to isolate the ESP32 from high-power components, ensuring safe operation when interacting with high-voltage devices like the solenoid valve.

Solenoid Valve: This valve is used to control the gas flow. When a gas leak or flame is detected, the relay triggers the solenoid valve to close, cutting off the gas supply for safety.

Buzzer and LED: The buzzer and LED function as immediate indicators of system status. The LED lights up to show gas leakage or flame detection, and the buzzer sounds an alarm to alert users of a dangerous situation.

Power Supply: A stable 5V to 12V power supply is used to power the entire system, including the ESP32, sensors, relay module, and solenoid valve, ensuring adequate power without overloading the system.

- 2.  Software Tools

Arduino IDE: The Arduino IDE is used for programming the ESP32. It provides an environment to write, compile, and upload code while offering various libraries to interface with sensors and external services like Telegram.

Telegram Bot: The Telegram bot is integrated to send notifications to the user about gas leaks or flame detection. It also provides a remote control feature for the solenoid valve, enabling the user to turn it on or off from the Telegram app.

- 3.  Libraries:

WiFi.h: This library is essential for enabling the ESP32 to connect to Wi-Fi, allowing the device to communicate with cloud-based services and send Telegram notifications.

WiFiClientSecure.h: This library provides secure HTTPS communication for the Telegram bot, ensuring safe message exchanges.

UniversalTelegramBot.h: This library simplifies interaction with the Telegram Bot API, allowing the ESP32 to send messages, receive commands, and handle user inputs.

# METHODOLOGY

The proposed Gas Leak Detection and Prevention System integrates hardware and software components to monitor environmental conditions, detect gas leakage and flame presence, and automatically control the gas flow via a solenoid valve. The system leverages the ESP32 microcontroller for real-time processing, sensor data acquisition, and communication with the user via a Telegram bot. The methodology includes sensor integration, software design, relay control, and Telegram notifications, ensuring an automated and reliable hazard detection mechanism.

- System Overview

The system is designed to perform two primary functions:

Gas Leak Detection: Monitor the concentration of gas using an MQ-5 sensor and automatically stop the gas flow if the concentration exceeds a predefined threshold.

Flame Detection: Monitor the presence of flame using an IR Flame Sensor. If the flame is detected, it prevents further gas flow and alerts the user.

Upon detecting either gas leakage or flame, the system triggers a relay module that controls the solenoid valve, cutting off the gas flow. The user is notified via Telegram, where they can also manually control the solenoid valve.

- Hardware Design

The hardware setup consists of the following components:

ESP32 Microcontroller: The ESP32 serves as the central processing unit, managing sensor readings and controlling outputs. It connects to Wi-Fi to enable Telegram communication.

MQ-5 Gas Sensor: This sensor detects the concentration of combustible gases such as LPG and natural gas. It provides an analog output, which is continuously monitored by the ESP32.

IR Flame Sensor: This sensor detects the presence of a flame by monitoring infrared radiation emitted by a flame. The digital output from the sensor is read by the ESP32.

Relay Module: The relay module is controlled by the ESP32 to switch the solenoid valve on and off. When a hazard is detected, the relay triggers the solenoid valve to close the gas supply line.

Solenoid Valve: The solenoid valve is triggered to stop the gas flow when the relay is activated, thus ensuring the safety of the environment.

Buzzer and LED (optional): These components are used for local signaling. The buzzer sounds and the LED lights up when a gas leak or flame presence is detected.

Circuit Connections:

- MQ-5 Sensor: Analog output connected to GPIO34 (ESP32).

- IR Flame Sensor: Digital output connected to GPIO14 (ESP32).

- Relay Module: Controlled via GPIO15 to manage the solenoid valve.

- Solenoid Valve: Powered through an external power supply and controlled via the relay.

- Buzzer/LED: Optional components connected to GPIO12 for local indication.

- Software Design

The software design includes several key components to process sensor data, perform hazard detection, and manage system control:

Sensor Data Acquisition:

  - The ESP32 continuously reads data from the MQ-5 and IR Flame sensors. The gas sensor provides an analog value that represents the gas concentration, and the flame sensor provides a digital signal indicating the presence or absence of a flame.

- Threshold Comparison:

  - A predefined gas concentration threshold is set to detect gas leakage. If the gas reading exceeds this threshold, the system triggers the solenoid valve to close, halting the gas flow.

  - Similarly, if the IR Flame Sensor detects a flame (output LOW), the system will stop the gas flow by controlling the relay.

- Relay Control:

  - The relay is triggered via GPIO15 of the ESP32, which controls the solenoid valve. When either gas leakage or flame presence is detected, the relay is activated to close the solenoid valve, preventing the flow of gas.

  - In normal operation (when no hazards are detected), the relay remains deactivated, and the gas flow is open.

- Telegram Integration:

  - A Telegram bot is created to allow remote monitoring and control of the system. The bot sends real-time notifications to the user when gas leakage or flame presence is detected.

  - The user can interact with the bot to manually control the solenoid valve by pressing a button within the Telegram interface. Additionally, the bot sends periodic updates every 15 minutes to notify the user of ongoing hazards.

- Periodic Notification:

  - The system sends periodic updates every 15 minutes if either gas leakage or flame presence persists. This keeps the user informed of the current status of the system.

- System Testing and Validation

The system is rigorously tested to ensure reliable operation under various conditions. Testing procedures include:

- Gas Leak Detection Test:

  - The system is exposed to controlled amounts of gas to test the sensor’s response. The solenoid valve is triggered when the gas concentration exceeds the predefined threshold, and the system sends a notification via Telegram.

- Flame Detection Test:

  - A flame source is introduced to test the IR Flame Sensor. When a flame is detected, the solenoid valve is closed, and the system sends a notification through Telegram.

- Telegram Notification Test:

  - The system sends notifications when either a gas leak or flame is detected. Periodic notifications are also tested to ensure they are sent every 15 minutes.

- Relay Control Test:

  - The relay control is tested by simulating gas leaks and flame detections, ensuring that the solenoid valve responds accordingly.

# Conclusion

The methodology ensures that the Gas Leak Detection and Prevention System effectively monitors environmental hazards and automatically controls the gas flow. The integration of sensors, real-time data processing, relay control, and Telegram notifications provides a robust solution for gas leak safety. The system is designed for both automation and manual user intervention, offering a comprehensive solution for gas leak prevention and flame detection.

# RESULT AND DISCUSSION

The Gas Leak Detection and Prevention System was developed and successfully tested in controlled conditions. The outcomes validate the system’s ability to detect gas leaks and open flames in real time, automatically shut off gas supply using a solenoid valve, and notify users through Telegram alerts. This section highlights the experimental results, user interaction performance, and key observations.

A. Gas Leak Detection Accuracy

The MQ-5 gas sensor responded reliably to the presence of LPG and methane. In a test environment, small leaks were simulated using a lighter (without flame), and the sensor detected gas concentrations exceeding the defined threshold (approx. 400 ppm).

- Gas detection response time: ~2–4 seconds after leak initiation

- Triggered actions: Relay activated, solenoid valve closed, buzzer sounded, and Telegram alert sent

- System reset occurred automatically once the gas concentration fell below the threshold

This behavior demonstrates the sensor’s responsiveness and the system’s fast reaction to potential hazards.

B. Flame Detection Performance

The IR flame sensor was tested by placing a lit match or lighter near the sensor. It accurately detected flames within a ~1 meter radius under normal lighting conditions.

- Detection latency: Less than 2 seconds

- Triggered actions: Telegram alert sent immediately, user prompted to use the ON/OFF button to control the solenoid valve manually via Telegram

This manual control for flame-related cases allows the user to assess whether the flame is intentional (e.g., cooking) or accidental, thus avoiding false shutoffs.

C. Telegram Bot Integration

The Telegram bot functioned effectively, providing:

- Real-time alerts for both gas leaks and flame detection

- Button controls for manually opening/closing the solenoid valve

- Periodic updates every 15 minutes only if hazardous conditions persisted

In user testing, response times for Telegram messages were consistently under 1 second after a trigger. The Telegram interface proved intuitive and lightweight, avoiding the need for a separate mobile application.

D. System Reliability and Uptime

The system was stress-tested over 24 hours of continuous operation. Key observations include:

- No false positives from gas or flame sensors under clean conditions

- Relay and solenoid valve functioned without failure across 20+ activation cycles

- Telegram continued to operate with uninterrupted message delivery

This confirms the suitability of the system for real-time and long-term deployment in household or small industrial scenarios.

E. Limitations

While the system is robust, a few limitations were identified:

- Sensor accuracy may be affected by dust or high humidity

- Manual intervention is needed to reactivate the solenoid after flame detection

- Power outage fallback is not yet implemented

These issues are considered for future enhancement through sensor calibration, automatic re-arming logic, and backup power integration.

# Conclusion

This paper presented the design and implementation of a real-time Gas Leak and Flame Detection System using the ESP32 microcontroller integrated with an MQ-5 gas sensor, IR flame sensor, solenoid valve, and Telegram bot interface. The system successfully detects hazardous conditions such as LPG leakage and open flames, and responds by triggering safety measures, including automatic gas shutoff via a solenoid valve and user alerts through Telegram.

The addition of flame detection addresses an important safety scenario—unattended gas stove flames—making the system more comprehensive. The manual valve control via Telegram in flame detection scenarios ensures flexibility and prevents unnecessary disruptions during safe usage (e.g., cooking). Real-time notifications and 15-minute periodic alerts during danger states enhance user awareness and intervention capability.

The proposed system offers a low-cost, scalable, and internet-connected solution suitable for homes, hostels, and small commercial kitchens. It eliminates the need for third-party apps like Blynk and relies entirely on Telegram for both communication and control, enhancing accessibility and user friendliness.

Future work may include:

Integration of battery backup to ensure functionality during power outages,

Addition of temperature sensing for fire intensity estimation,

Cloud-based data logging and analytics for long-term safety monitoring.

Overall, the system contributes to domestic safety automation and provides a foundation for smart IoT-based fire and gas hazard prevention systems.

##### Acknowledgment

##### We would like to thank our Project Guide Prof. Archana Chaudhari for sharing their pearls of wisdom and guiding us and for completing this project.

##### References

[1] Singh, A., et al. (2023). LPG Gas Leakage Detection using ESP32. ISJEM Journal.

[2] Ramesh, P., et al. (2022). IoT-Based Gas Leakage Monitoring Systems: A Review. IJERT.

[3] Kiran, R., et al. (2023). Integration of MQ-5 Gas Sensors with Arduino Board. ResearchGate.

[4] Islam, M., et al. (2021). Detection of LPG with MQ-5 Sensor and Microcontrollers. IEEE Access.

[5] Zhang, Y., et al. (2022). Flame Detection via Infrared Spectrum Optimization. ScienceDirect.

[6] Babrauskas, V. (2003). Review of Fire Detection Technologies. ResearchGate.

[7] Sharma, R., et al. (2024). ESP32-Based Detection with Blynk Integration. ResearchGate.

[8] Kumar, S., et al. (2023). A Review of IoT-Based Gas Detection with Mobile Integration. IJIRCCE.

[9] Patel, D., et al. (2021). Automatic Gas and Flame Detection System Using MQ-5 Sensor. IJRASET.

[10] Kaur, G., et al. (2022). Dual-Sensor Fire and Gas Monitoring Using NodeMCU. IJERT.

[11] Gupta, P., et al. (2023). Telegram-Bot Based Alert System for Gas Leakage. ResearchGate.

[12] Ahmed, S., et al. (2022). Telegram Notification for IoT Alert Systems. IJAREEIE.

[13] Lee, J., et al. (2023). IoT Alerting Using Messaging APIs: A Comparative Study. ACM IoTConf.

[14] Johnson, M., et al. (2022). Mobile Interfaces for IoT-Based Fire Monitoring. IEEE Sensors Journal.

[15] Sahu, A., et al. (2023). Solenoid Valve Automation in Gas Leak Scenarios. Elsevier Procedia CS.

[16] Das, T., et al. (2023). Relay-Controlled Gas Safety Systems in Smart Homes. IJRTE.

[17] Mishra, R., et al. (2021). A Survey of Multi-Sensor Fusion in Fire Detection. ScienceDirect.

[18] Chen, L., et al. (2024). Deep Learning Techniques for Fire and Smoke Detection. Elsevier.

[19] Park, Y., et al. (2023). Vision-Based Smart Fire Monitoring System. MDPI Sensors.

- [20] Kim, S., et al. (2022). IoT-Based Automation for Real-Time Hazard Mitigation. IEEE Access.